SECTION 09 29 00
GYPSUM BOARD

PART 1 - GENERAL

1.1 DESCRIPTION

This section specifies installation and finishing of gypsum board.

1.2 RELATED WORK

A.	Installation of steel framing members for walls, partitions, furring, soffits, and ceilings: Section 09 22 16, NON-STRUCTURAL METAL FRAMING.

B.	Sound deadening board: Section 07 21 13, THERMAL INSULATION.

C.	Acoustical Sealants: Section 07 92 00, JOINT SEALANTS.

1.3 TERMINOLOGY

A.	Definitions and description of terms shall be in accordance with ASTM C11, C840, and as specified.

B.	Underside of Structure Overhead: In spaces where steel trusses or bar joists are shown, the underside of structure overhead shall be the underside of the floor or roof construction supported by the trusses or bar joists.

C.	"Yoked": Gypsum board cut out for opening with no joint at the opening (along door jamb or above the door).

1.4 SUBMITTALS

A.	Submit in accordance with Section 01 33 23, SHOP DRAWINGS, PRODUCT DATA, AND SAMPLES.

B.	Manufacturer's Literature and Data:

1.	Cornerbead and edge trim.

2.	Finishing materials.

3.	Laminating adhesive.

4.	Gypsum board, each type.

C.	Shop Drawings:

1.	Typical gypsum board installation, showing corner details, edge trim details and the like.

2.	Typical sound rated assembly, showing treatment at perimeter of partitions and penetrations at gypsum board.

3.	Typical fire rated assembly and column fireproofing, indicating details of construction same as that used in fire rating test.

D.	Samples:

1.	Cornerbead.

2.	Edge trim.

3.	Control joints.

E.	Test Results:

1.	Fire rating test, each fire rating required for each assembly.

2.	Sound rating test.

F.	Certificates: Certify that gypsum board types, gypsum backing board types, cementitious backer units, and joint treating materials do not contain asbestos material.

1.5 DELIVERY, IDENTIFICATION, HANDLING AND STORAGE

In accordance with the requirements of ASTM C840.

1.6 ENVIRONMENTAL CONDITIONS

In accordance with the requirements of ASTM C840.

1.7 APPLICABLE PUBLICATIONS

A.	The publications listed below form a part of this specification to the extent referenced. The publications are referenced in the text by the basic designation only.

B.	American Society for Testing And Materials (ASTM):

C11-08	Terminology Relating to Gypsum and Related Building Materials and Systems

C475-02	Joint Compound and Joint Tape for Finishing Gypsum Board

C840-08	Application and Finishing of Gypsum Board

C919-08	Sealants in Acoustical Applications

C954-07	Steel Drill Screws for the Application of Gypsum Board or Metal Plaster Bases to Steel Stud from 0.033 in. (0.84mm) to 0.112 in. (2.84mm) in thickness

C1002-07	Steel Self-Piercing Tapping Screws for the Application of Gypsum Panel Products or Metal Plaster Bases to Wood Studs or Steel Studs

C1047-05	Accessories for Gypsum Wallboard and Gypsum Veneer Base

C1177-06	Glass Mat Gypsum Substrate for Use as Sheathing

C1658-06	Glass Mat Gypsum Panels

C1396-06	Gypsum Board

E84-08	Surface Burning Characteristics of Building Materials

C.	Underwriters Laboratories Inc. (UL):

Latest Edition	Fire Resistance Directory

D.	Inchcape Testing Services (ITS):

Latest Editions	Certification Listings

PART 2 - PRODUCTS

2.1 GYPSUM BOARD

A.	Gypsum Board: ASTM C1396, Type X, 16 mm (5/8 inch) thick unless shown otherwise. Shall contain a minimum of 20 percent recycled gypsum.

B.	Water Resistant Gypsum Backing Board: ASTM C620, Type X, 16 mm (5/8 inch) thick.

C.	Gypsum cores shall contain maximum percentage of postindustrial recycled gypsum content available in the area (a minimum of 95 percent postindustrial recycled gypsum content). Paper facings shall contain 100 percent post-consumer recycled paper content.

2.2 ACCESSORIES

A.	ASTM C1047, except form of 0.39 mm (0.015 inch) thick zinc coated steel sheet or rigid PVC plastic.

B.	Flanges not less than 22 mm (7/8 inch) wide with punchouts or deformations as required to provide compound bond.

2.3 FASTENERS

A.	ASTM C1002 and ASTM C840, except as otherwise specified.

B.	ASTM C954, for steel studs thicker than 0.04 mm (0.33 inch).

C.	Select screws of size and type recommended by the manufacturer of the material being fastened.

D.	For fire rated construction, type and size same as used in fire rating test.

E.	Clips: Zinc-coated (galvanized) steel; gypsum board manufacturer's standard items.

2.4 FINISHING MATERIALS AND LAMINATING ADHESIVE

ASTM C475 and ASTM C840. Free of antifreeze, vinyl adhesives, preservatives, biocides and other VOC. Adhesive shall contain a maximum VOC content of 50 g/l.

PART 3 - EXECUTION

3.1 GYPSUM BOARD HEIGHTS

A.	Extend all layers of gypsum board from floor to underside of structure overhead on following partitions and furring:

1.	Two sides of partitions:

a.	Fire rated partitions.

b.	Smoke partitions.

c.	Sound rated partitions.

d.	Full height partitions shown (FHP).

2.	One side of partitions or furring:

a.	Inside of exterior wall furring or stud construction.

b.	Room side of room without suspended ceilings.

c.	Furring for pipes and duct shafts, except where fire rated shaft wall construction is shown.

3.	Extend all layers of gypsum board construction used for fireproofing of columns from floor to underside of structure overhead, unless shown otherwise.

B.	In locations other than those specified, extend gypsum board from floor to heights as follows:

1.	Not less than 100 mm (4 inches) above suspended acoustical ceilings.

2.	At ceiling of suspended gypsum board ceilings.

3.	At existing ceilings.

3.2 INSTALLING GYPSUM BOARD

A.	Coordinate installation of gypsum board with other trades and related work.

B.	Install gypsum board in accordance with ASTM C840, except as otherwise specified.

C.	Moisture and Mold–Resistant Assemblies: Provide and install moisture and mold-resistant glass mat gypsum wallboard products with moisture-resistant surfaces complying with ASTM C1658 where shown and in locations which might be subject to moisture exposure during construction.

D.	Use gypsum boards in maximum practical lengths to minimize number of end joints.

E.	Bring gypsum board into contact, but do not force into place.

F.	Ceilings:

1.	For single-ply construction, use perpendicular application.

2.	For two-ply assembles:

a.	Use perpendicular application.

b.	Apply face ply of gypsum board so that joints of face ply do not occur at joints of base ply with joints over framing members.

G.	Walls (Except Shaft Walls):

1.	When gypsum board is installed parallel to framing members, space fasteners 300 mm (12 inches) on center in field of the board, and 200 mm (8 inches) on center along edges.

2.	When gypsum board is installed perpendicular to framing members, space fasteners 300 mm (12 inches) on center in field and along edges.

3.	Stagger screws on abutting edges or ends.

4.	For single-ply construction, apply gypsum board with long dimension either parallel or perpendicular to framing members as required to minimize number of joints except gypsum board shall be applied vertically over "Z" furring channels.

5.	For two-ply gypsum board assemblies, apply base ply of gypsum board to assure minimum number of joints in face layer. Apply face ply of wallboard to base ply so that joints of face ply do not occur at joints of base ply with joints over framing members.

6.	For three-ply gypsum board assemblies, apply plies in same manner as for two-ply assemblies, except that heads of fasteners need only be driven flush with surface for first and second plies. Apply third ply of wallboard in same manner as second ply of two-ply assembly, except use fasteners of sufficient length enough to have the same penetration into framing members as required for two-ply assemblies.

7.	No offset in exposed face of walls and partitions will be permitted because of single-ply and two-ply or three-ply application requirements.

H. Electrical and Telecommunications Boxes:

1. Seal annular spaces between electrical and telecommunications receptacle boxes and gypsum board partitions.

I.	Accessories:

1.	Set accessories plumb, level and true to line, neatly mitered at corners and intersections, and securely attach to supporting surfaces as specified.

2.	Install in one piece, without the limits of the longest commercially available lengths.

3.	Corner Beads:

a.	Install at all vertical and horizontal external corners and where shown.

b.	Use screws only.  Do not use crimping tool.

4.	Edge Trim (casings Beads):

a.	At both sides of expansion and control joints unless shown otherwise.

b.	Where gypsum board terminates against dissimilar materials and at perimeter of openings, except where covered by flanges, casings or permanently built-in equipment.

c.	Where gypsum board surfaces of non-load bearing assemblies abut load bearing members.

d.	Where shown.

3.3 FINISHING OF GYPSUM BOARD

A.	Finish joints, edges, corners, and fastener heads in accordance with ASTM C840. Use Level 4 finish for al finished areas open to public view.

B.	Before proceeding with installation of finishing materials, assure the following:

1.	Gypsum board is fastened and held close to framing or furring.

2.	Fastening heads in gypsum board are slightly below surface in dimple formed by driving tool.

C.	Finish joints, fasteners, and all openings, including openings around penetrations, on that part of the gypsum board extending above suspended ceilings to seal surface of non decorated smoke barrier, fire rated, and sound rated gypsum board construction.  After the installation of hanger rods, hanger wires, supports, equipment, conduits, piping and similar work, seal remaining openings and maintain the integrity of the smoke barrier, fire rated and sound rated construction. Sanding is not required of non decorated surfaces.

3.4 REPAIRS

A.	After taping and finishing has been completed, and before decoration, repair all damaged and defective work, including non-decorated surfaces.

B.	Patch holes or openings 13 mm (1/2 inch) or less in diameter, or equivalent size, with a setting type finishing compound or patching plaster.

C.	Repair holes or openings over 13 mm (1/2 inch) diameter, or equivalent size, with 16 mm (5/8 inch) thick gypsum board secured in such a manner as to provide solid substrate equivalent to undamaged surface.

D.	Tape and refinish scratched, abraded or damaged finish surfaces including cracks and joints in non decorated surface to provide smoke tight construction, fire protection equivalent to the fire rated construction, and STC equivalent to the sound rated construction.

3.5 unaccessible ceilings

At Mental Health and Behavioral Nursing Units, areas accessible to patients and not continuously observable by staff (e.g., patient bedrooms, day rooms), ceilings should be a solid material such as gypsum board. This will limit patient access. Access doors are needed to access electrical and mechanical equipment above the ceiling.  These doors should be locked to prevent unauthorized access and secured to ceiling using tamper resistant fasteners.

- - - E N D - - -